State of Idaho

Cross Connection Requirements

IDAHO AMINISTRATIVE CODE		IDAPA 58.01.08

Department of Environmental Quality		Public Drinking Water systems

- An inspection once a year of all facilities listed in subsection 900.02 (Table 2) to  locate cross connections and determine required suitable protection.

- Required installation and operation of adequate backflow prevention devices.  A list of minimum recommended devices for various facilities is provided in sub section 900.02 (Table 2).

- Annual inspections by American water Works Association-certified tester, or equal, of all installed backflow prevention devices to insure operability.

- Discontinuance of service to any facility where suitable backflow protection has not been provided for a cross connection.

- If double check valves and/or reduced pressure principle backflow prevention       devices are used, they must pass a performance test conducted by the University of Southern California Foundation for Cross Connection Control and Hydraulic        Research or meet American Water Works Association C-510 or C-511 standard or another equal test approved by the Department.

- If atmospheric vacuum breakers and pressure vacuum breakers are used, they shall be marked approved by the International Association of Plumbing and Mechanical Officials (IAPMO) or by the American Society of Sanitation Engineers (ASSE).

- Resilient seated shutoff valves shall be used after the effective date of these rules when double check valves, reduced pressure backflow prevention devices, and    pressure vacuum breakers are installed.